Multidisciplinary Characterization of Uranium Mine Water: U Bioremediation Perspectives

Newman-Portela A. M.1,2, Krawczyk-Bärsch E.2, Lopez-Fernandez M.1, Bok F.2, Kassahun A.3, Raff J. 2, Merroun M. L.1

1Department of Microbiology, Faculty of Science, University of Granada (Granada, Spain)

antnewpor@ugr.es

2Department of Biogeochemistry, Institute of Resource Ecology, Helmholtz-Zentrum Dresden-Rossendorf

(Germany)

3WISMUT GmbH (Chemnitz, Germany)

___________________________________________________________________________

In Saxony and Thuringia (Germany), intensive uranium mining took place for decades until 1990. These intensive mining activities have left many U contaminated areas. The resulting release of the soluble U into the mine water represents a major health risk. Remediation approaches using indigenous microbial communities are an efficient strategy [1,2]. In this study, we have characterized the microbial diversity and geochemistry of water samples from two German former uranium mines (Schlema-Alberoda and Pöhla) to design bioremediation approach based on U enzymatic reduction.

ICP-MS and Ion-Chromatography studies showed that the mine water exhibited a higher concentration of U, sulfate, iron and manganese in Schlema-Alberoda compared to that of Pöhla (U: 1.01 and 0.11mg/L, sulfate: 335 and 0.26mg/L, iron: 0.99 and 0.13mg/L and manganese: 1.44 and 0.16mg/L, respectively). TRLFS studies identified an aqueous uranyl carbonate species [UO2(CO3)3-4]. The 16S and ITS1 rRNA gene sequencing of both mine waters revealed a high microbial diversity. The total bacterial community composition combining both mine waters indicated an average relative abundance of sulfate-reducing-bacteria (e.g., Desulfovibrio) and iron-oxidizing-bacteria (e.g., Gallionella, Sideroxydans). These bacterial groups are reported to be involved in U(VI) reduction as a key process in the bioremediation of anoxic U contaminated sites [2].

To design a bioremediation strategy for these U-contaminated mine water, Schlema-Alberoda water was used as a reference for setting anoxic microcosms. Concretely, U-reducing-bacteria were biostimulated by glycerol (10mM) as electron donor. ICP-MS and Ion-Chromatography analysis from the microcosms revealed a decrease of U (≈90%), sulfate (≈60%), iron (≈86%) and manganese (≈88%). In addition, a drop of Eh and pH of the system was detected. A theoretical thermodynamic Eh-pH predominance diagram was calculated by Geochemist ́s Workbench, indicating the formation of U(IV) precipitates, probably uraninite, after 3 months at the latest. Finally, U(IV) was detected by UV-Visible in the precipitate at the end of the experiment.

These results show that the U reduction of soluble U(VI) to insoluble U(IV) is favoured by the addition of an electron donor (glycerol) in low concentrated U-contaminated mines water. Therefore, this strategy might be an efficient bioremediation approach relevant for U-contaminated waters, by biostimulating their native microbial community.

References:

[1] Krawczyk-Bärsch (2018) J. Hazard. Mater. 347:233–241

[2] Lopez-Fernandez (2020). Microb. Biotechnol. 1–19